MLAG Configuration Guide

Multi-Chassis Link Aggregation for PicOS Switches

Immunocore IT Infrastructure  |  CONFIDENTIAL

# 1. Overview

MLAG (Multi-chassis Link Aggregation Group) deploys LAG technology across a pair of switches that appear as a single device to any downstream third device at Layer 2. The two MLAG peer switches share the same Domain ID and maintain synchronisation by exchanging control plane messages and MAC address learning over the peer link using L2 multicast packets.

The downstream device can be any endpoint — an L2 switch or a server — that supports LACP link aggregation. It has no awareness that it is connected to two separate physical switches.

## 1.1 Key Benefits

| Benefit | Description |
| --- | --- |
| Increased Bandwidth | Aggregates multiple Ethernet ports across two switches. Maximum bandwidth equals the sum of all member port bandwidths. |
| Higher Reliability | Dual-working mechanism — if a link or device fails, traffic automatically switches to the remaining available member links. |
| Load Balancing | Active-active load balancing across all member links within the MLAG domain. |

# 2. Architecture & Key Concepts

## 2.1 MLAG Components

| Term | Description |
| --- | --- |
| MLAG Domain | Logical grouping of two peer switches sharing the same domain ID (1–255). Look up the domain ID to use in Immunocore - IT Network v3 CONFIDENTIAL.xlsx. |
| Node 0 / Node 1 | Identity of each switch within the MLAG domain. One peer is node 0, the other is node 1. |
| Peer Link (ae) | The LAG interface connecting the two MLAG peer switches. Carries both control (VLAN 4088) and data plane traffic. |
| VLAN 4088 | Reserved MLAG peer link VLAN used for control plane sync between the two nodes. |
| MLAG Link ID | A shared identifier that ties two ae interfaces (one on each peer) into a single logical MLAG. Must be identical on both peers. |
| Downstream ae | The LAG interface facing a downstream server or switch. Each downstream device gets a unique MLAG link ID. |

Important: VLAN 4088 Peer Link Trunk The peer link ae must carry ALL VLANs that will traverse the MLAG domain — not just VLAN 4088. If a downstream link fails, traffic must traverse the peer link to reach the active member on the other switch. Use vlan members 1-4094 or explicitly list all VLANs carried by any MLAG-attached ae.

Important: MLAG Link ID The link ID must be identical on both MLAG peer switches for the same downstream device. The ae number is local to each switch and may differ, but the link ID is the shared identifier that MLAG uses to pair the two physical ports into a single logical link.

# 3. Useful Verification Commands

| Command | Purpose |
| --- | --- |
| run show mlag domain summary | Shows MLAG domain state, peer connectivity, and node role. |
| run show mlag link summary | Shows all MLAG links, their link IDs, and status. |
| run show mac-address table | Verifies MAC synchronisation across peers. |
| run show lacp neighbor | Confirms LACP negotiation with downstream devices. |
| run show interface aggregate-ethernet ae99 | Checks ae interface status and member ports. |
| run show copp statistics forwarding-class lacp-class | Verifies LACP control plane packet handling. |

# 4. Peer Link Configuration

The peer link connects the two MLAG peer switches and must be configured identically (except for node role and IP address) on both switches. The peer link ae carries VLAN 4088 as its native VLAN plus all other VLANs in the MLAG domain.

## 4.1 Node 0 Configuration

Replace {x} with your domain ID, {y} with your subnet octet, {a} with port type prefix, and {b} with port numbers.

# ─── VLAN 4088 - MLAG Peer Link ───────────────────────────────────────────

set vlans vlan-id 4088 description "MLAG Peer Link"

set vlans vlan-id 4088 l3-interface "vlan4088"

set l3-interface vlan-interface vlan4088 address 10.{y}.161.1 prefix-length 24

# ─── MLAG Domain ────────────────────────────────────────────────────────────

set protocols mlag domain {x} node 0

set protocols mlag domain {x} peer-ip 10.{y}.161.2 peer-link "ae1"

set protocols mlag domain {x} peer-ip 10.{y}.161.2 peer-vlan 4088

# ─── Peer Link ae1 ──────────────────────────────────────────────────────────

set interface aggregate-ethernet ae1 aggregated-ether-options lacp enable true

set interface aggregate-ethernet ae1 family ethernet-switching native-vlan-id 4088

set interface aggregate-ethernet ae1 family ethernet-switching port-mode "trunk"

set interface aggregate-ethernet ae1 family ethernet-switching vlan members 1-4094

# ─── Physical member ports (amend prefix {a} and port {b} for your hardware) ─

set interface gigabit-ethernet {a}-1/1/{b} ether-options 802.3ad "ae1"

set interface gigabit-ethernet {a}-1/1/{b} ether-options 802.3ad "ae1"

## 4.2 Node 1 Configuration

# ─── VLAN 4088 - MLAG Peer Link ───────────────────────────────────────────

set vlans vlan-id 4088 description "MLAG Peer Link"

set vlans vlan-id 4088 l3-interface "vlan4088"

set l3-interface vlan-interface vlan4088 address 10.{y}.161.2 prefix-length 24

# ─── MLAG Domain ────────────────────────────────────────────────────────────

set protocols mlag domain {x} node 1

set protocols mlag domain {x} peer-ip 10.{y}.161.1 peer-link "ae1"

set protocols mlag domain {x} peer-ip 10.{y}.161.1 peer-vlan 4088

# ─── Peer Link ae1 ──────────────────────────────────────────────────────────

set interface aggregate-ethernet ae1 aggregated-ether-options lacp enable true

set interface aggregate-ethernet ae1 family ethernet-switching native-vlan-id 4088

set interface aggregate-ethernet ae1 family ethernet-switching port-mode "trunk"

set interface aggregate-ethernet ae1 family ethernet-switching vlan members 1-4094

# ─── Physical member ports (amend prefix {a} and port {b} for your hardware) ─

set interface gigabit-ethernet {a}-1/1/{b} ether-options 802.3ad "ae1"

set interface gigabit-ethernet {a}-1/1/{b} ether-options 802.3ad "ae1"

Definitely desirable someone that can lead technical transformation (ongoing after suite is 100%) rather than just oversee management, as there is no l3 technical staff the role currently manages.

Cloud/Interconnect system settings

# 5. Downstream Device Configuration

A downstream device is any endpoint — L2 switch or server — that connects one port to each MLAG peer switch and presents a single logical LAG to the network. The downstream device must use LACP (802.3ad).

## 5.1 Downstream Switch or Server (Generic Template)

Replace {ae} with your ae number, adjust native VLAN and VLAN members to match the downstream device's requirements.

set interface aggregate-ethernet ae{99} aggregated-ether-options lacp enable true

set interface aggregate-ethernet ae{99} family ethernet-switching native-vlan-id {249}

set interface aggregate-ethernet ae{99} family ethernet-switching port-mode "trunk"

set interface aggregate-ethernet ae{99} family ethernet-switching vlan members {1-254}

# ─── Physical member ports ──────────────────────────────────────────────────

# Note: port prefix (xe, et, ge) is hardware-specific — verify with: run show interface

set interface gigabit-ethernet {a}-1/1/{b} ether-options 802.3ad "ae{99}"

set interface gigabit-ethernet {a}-1/1/{b} ether-options 802.3ad "ae{99}"

## 5.2 Dual-Homed Server (One NIC Port Per MLAG Peer)

This is the standard dual-homed server topology. One NIC port connects to Node 0, the other to Node 1. The server bonds both ports with LACP and sees a single logical interface. From the server's perspective there is no awareness of two separate switches.

Topology Server NIC Port 1 (eth0) ──── Node 0 (e.g. xe-0/1/5) NIC Port 2 (eth1) ──── Node 1 (e.g. xe-0/1/5) bond0 (LACP 802.3ad) ─ single logical bond

### Node 0 — Server-Facing ae

# ─── ae for server-facing link ─────────────────────────────────────────────

set interface aggregate-ethernet ae10 aggregated-ether-options lacp enable true

set interface aggregate-ethernet ae10 family ethernet-switching native-vlan-id 100

set interface aggregate-ethernet ae10 family ethernet-switching port-mode "trunk"

set interface aggregate-ethernet ae10 family ethernet-switching vlan members 100,200

# ─── Physical port ──────────────────────────────────────────────────────────

set interface gigabit-ethernet xe-0/1/5 ether-options 802.3ad "ae10"

# ─── Bind ae10 to MLAG domain — link ID MUST match Node 1 ──────────────────

set protocols mlag domain 3 interface ae10 link 5

### Node 1 — Server-Facing ae

# ─── ae for server-facing link (ae number may differ per switch) ───────────

set interface aggregate-ethernet ae10 aggregated-ether-options lacp enable true

set interface aggregate-ethernet ae10 family ethernet-switching native-vlan-id 100

set interface aggregate-ethernet ae10 family ethernet-switching port-mode "trunk"

set interface aggregate-ethernet ae10 family ethernet-switching vlan members 100,200

# ─── Physical port ──────────────────────────────────────────────────────────

set interface gigabit-ethernet xe-0/1/5 ether-options 802.3ad "ae10"

# ─── Same link ID as Node 0 — this ties both halves together ────────────────

set protocols mlag domain 3 interface ae10 link 5

### Server Side — Linux Bond (802.3ad)

# /etc/network/interfaces or equivalent (nmcli / netplan)

bond0:

mode: 802.3ad # LACP — required, static LAG will not work with MLAG

slaves:

- eth0 # NIC port → Node 0

- eth1 # NIC port → Node 1

# 6. MLAG Link Binding

Once the MLAG domain and peer link are configured, each downstream ae must be bound to an MLAG link ID. This tells MLAG which ae on Node 0 and which ae on Node 1 form the two halves of the same logical link.

# Syntax:

set protocols mlag domain <domain-id> interface <ae> link <link-id>

# Example — bind ae3 to MLAG link 2 of domain 3:

set protocols mlag domain 3 interface ae3 link 2

# Example — multiple downstream devices in the same domain:

set protocols mlag domain 3 interface ae3 link 1 # downstream device A

set protocols mlag domain 3 interface ae5 link 2 # downstream device B

set protocols mlag domain 3 interface ae7 link 3 # downstream device C

Link ID Rules • Link IDs must be identical on both peer nodes for the same downstream device. • Each MLAG (downstream device) gets a unique link ID within the domain. • The ae number is local to each switch and can differ between peers — the link ID is the shared identifier. • VLAN configuration on both ae interfaces must be identical — mismatched VLANs cause inconsistent traffic behaviour.

# 7. Example: Storage MLAG with VRRP

The following is a working example of an MLAG domain for storage switches (STOR01/STOR02) using VLAN 48 and 49 for storage paths, with VRRP providing gateway redundancy on each storage VLAN.

## 7.1 STOR01 — Node 0

# ─── VLAN 4088 Peer Link ────────────────────────────────────────────────────

set vlans vlan-id 4088 description "MLAG Peer Link"

set vlans vlan-id 4088 l3-interface "vlan4088"

set l3-interface vlan-interface vlan4088 address 10.12.161.1 prefix-length 24

# ─── MLAG Domain 6 ──────────────────────────────────────────────────────────

set protocols mlag domain 6 node 0

set protocols mlag domain 6 peer-ip 10.12.161.2 peer-link "ae6"

set protocols mlag domain 6 peer-ip 10.12.161.2 peer-vlan 4088

# ─── Peer Link ae6 ──────────────────────────────────────────────────────────

set interface aggregate-ethernet ae6 aggregated-ether-options lacp enable true

set interface aggregate-ethernet ae6 family ethernet-switching native-vlan-id 4088

set interface aggregate-ethernet ae6 family ethernet-switching port-mode "trunk"

set interface aggregate-ethernet ae6 family ethernet-switching vlan members 48,49

set interface gigabit-ethernet xe-1/1/x ether-options 802.3ad "ae6"

set interface gigabit-ethernet xe-1/1/x ether-options 802.3ad "ae6"

# ─── Storage VLANs ───────────────────────────────────────────────────────────

set vlans vlan-id 48 description "Storage_Path_A"

set vlans vlan-id 48 l3-interface "vlan-48"

set vlans vlan-id 49 description "Storage_Path_B"

set vlans vlan-id 49 l3-interface "vlan-49"

set l3-interface vlan-interface vlan-48 address 10.12.48.1 prefix-length 24

set l3-interface vlan-interface vlan-49 address 10.12.49.1 prefix-length 24

# ─── VRRP (Node 0 is primary — priority 110) ────────────────────────────────

set protocols vrrp interface vlan-48 vrid 48 version 3

set protocols vrrp interface vlan-48 vrid 48 ip 10.12.48.254

set protocols vrrp interface vlan-48 vrid 48 priority 110

set protocols vrrp interface vlan-48 vrid 48 load-balance disable false

set protocols vrrp interface vlan-49 vrid 49 version 3

set protocols vrrp interface vlan-49 vrid 49 ip 10.12.49.254

set protocols vrrp interface vlan-49 vrid 49 priority 110

set protocols vrrp interface vlan-49 vrid 49 load-balance disable false

## 7.2 STOR02 — Node 1

# ─── VLAN 4088 Peer Link ────────────────────────────────────────────────────

set vlans vlan-id 4088 description "MLAG Peer Link"

set vlans vlan-id 4088 l3-interface "vlan4088"

set l3-interface vlan-interface vlan4088 address 10.12.161.2 prefix-length 24

# ─── MLAG Domain 6 ──────────────────────────────────────────────────────────

set protocols mlag domain 6 node 1

set protocols mlag domain 6 peer-ip 10.12.161.1 peer-link "ae6"

set protocols mlag domain 6 peer-ip 10.12.161.1 peer-vlan 4088

# ─── Peer Link ae6 ──────────────────────────────────────────────────────────

set interface aggregate-ethernet ae6 aggregated-ether-options lacp enable true

set interface aggregate-ethernet ae6 family ethernet-switching native-vlan-id 4088

set interface aggregate-ethernet ae6 family ethernet-switching port-mode "trunk"

set interface aggregate-ethernet ae6 family ethernet-switching vlan members 48,49

set interface gigabit-ethernet xe-1/1/x ether-options 802.3ad "ae6"

set interface gigabit-ethernet xe-1/1/x ether-options 802.3ad "ae6"

# ─── Storage VLANs ───────────────────────────────────────────────────────────

set vlans vlan-id 48 description "Storage_Path_A"

set vlans vlan-id 48 l3-interface "vlan-48"

set vlans vlan-id 49 description "Storage_Path_B"

set vlans vlan-id 49 l3-interface "vlan-49"

set l3-interface vlan-interface vlan-48 address 10.12.48.2 prefix-length 24

set l3-interface vlan-interface vlan-49 address 10.12.49.2 prefix-length 24

# ─── VRRP (Node 1 is secondary — priority 100) ──────────────────────────────

set protocols vrrp interface vlan-48 vrid 48 version 3

set protocols vrrp interface vlan-48 vrid 48 ip 10.12.48.254

set protocols vrrp interface vlan-48 vrid 48 priority 100

set protocols vrrp interface vlan-48 vrid 48 load-balance disable false

set protocols vrrp interface vlan-49 vrid 49 version 3

set protocols vrrp interface vlan-49 vrid 49 ip 10.12.49.254

set protocols vrrp interface vlan-49 vrid 49 priority 100

set protocols vrrp interface vlan-49 vrid 49 load-balance disable false

# 8. Configuration Checklist

| Step | Task |
| --- | --- |
| 1 | Confirm domain ID from Immunocore - IT Network v3 CONFIDENTIAL.xlsx |
| 2 | Assign peer link IPs — Node 0 gets .1, Node 1 gets .2 (10.{y}.161.x/24) |
| 3 | Configure VLAN 4088 and vlan4088 L3 interface on both nodes |
| 4 | Set node role (node 0 / node 1) on each peer |
| 5 | Configure peer-ip and peer-link pointing to the other node's ae |
| 6 | Configure peer link ae — native VLAN 4088, trunk, vlan members 1-4094 |
| 7 | Bind physical ports into the peer link ae on both nodes |
| 8 | Configure downstream ae on each node — native VLAN, trunk, VLAN members |
| 9 | Bind physical downstream ports into their respective ae |
| 10 | Bind each downstream ae to its MLAG link ID (same ID on both nodes) |
| 11 | Verify: run show mlag domain summary — check domain is UP |
| 12 | Verify: run show mlag link summary — check all links are active |
| 13 | Verify: run show lacp neighbor — confirm LACP negotiation downstream |
| 14 | Verify: run show mac-address table — confirm MAC sync across peers |

# 9. Common Pitfalls

| Issue | Resolution |
| --- | --- |
| MLAG link not forming | Check link ID matches on both peers. Verify LACP is enabled on the downstream device. |
| Traffic black-holing on failover | Ensure the peer link ae carries all necessary VLANs (1-4094 or explicit list). |
| MAC flapping / loop | Confirm only two physical ports are bonded per ae. Check STP is not interfering. |
| VLAN mismatch | Both nodes must have identical VLAN config on each downstream ae — check native VLAN and members. |
| VRRP not electing primary | Confirm priority is higher on the intended primary node (110 vs 100). |
| Port label error | Port prefix (xe, et, ge) is hardware-specific. Use run show interface to confirm. |

# 10. eBGP 4-Router Dual-Homed Server Design

This section covers the design for connecting servers with up to 4 NICs into 4 independent routers using eBGP, providing maximum bandwidth through active-active ECMP across all paths and full resilience through independent BGP sessions per NIC.

## 10.1 Design Principles

| Principle | Detail |
| --- | --- |
| One AS per building | All servers in a building share one AS number. Servers are differentiated by their unique loopback /32 used as BGP router-id. |
| 4 independent sessions | Each NIC peers with a separate router via its own eBGP session. No bonding or LACP — each NIC is a standalone routed interface. |
| ECMP active-active | All 4 paths carry traffic simultaneously, maximising available bandwidth across all NICs. |
| Loopback as identity | The server&#x2019;s /32 loopback is its single stable IP. Clients always connect to this address — never to NIC IPs. |
| allow-as-in 1 | Configured on all routers to accept routes even if the server&#x2019;s AS appears in the path, preventing false loop detection. |
| VRRP for non-BGP traffic | VRRP runs across all 4 routers per server VLAN providing gateway redundancy for any traffic not using BGP. |

## 10.2 AS Number Allocation

Each building has a dedicated server AS number from the 65499-65519 range. All servers within a building share this AS — only the loopback IP (BGP router-id) differs per server.

| Building | Server AS |
| --- | --- |
| MEP-91 | 65499 |
| MEP-92 | 65500 |
| MEP-93 | 65501 |
| MEP-94 | 65502 |
| MEP-96 | 65503 |
| GBG (Gaithersburg) | 65504 |
| RAD (Radnor) | 65505 |
| UK-RES01 | 65506 |
| UK-RES02 | 65507 |
| EU-RES01 through EU-RES04 | 65508 - 65511 |
| US-RES01 through US-RES04 | 65512 - 65515 |
| EXP-01 through EXP-04 | 65516 - 65519 |
| Reserved buffer | 65520 - 65534 (15 ASNs) |

## 10.3 IP Addressing

| Block | Purpose |
| --- | --- |
| 10.240.0.0/16 | Server P2P links. One /21 per building, split into 4 x /23 (one per router). Each /23 provides 126 x /30 point-to-point subnets. |
| 10.249.0.0/16 | Server loopbacks. One /24 per building. Each server gets a unique /32 used as its BGP router-id and stable client-facing identity. |

P2P Subnet Structure per Building Each building gets a /21 from 10.240.0.0/16, split across the 4 routers: Core01 (VRRP 120) → First /23 e.g. 10.240.0.0/23 Router IP: 10.240.0.1 Core02 (VRRP 110) → Second /23 e.g. 10.240.2.0/23 Router IP: 10.240.2.1 Res03 (VRRP 100) → Third /23 e.g. 10.240.4.0/23 Router IP: 10.240.4.1 Res04 (VRRP 90) → Fourth /23 e.g. 10.240.6.0/23 Router IP: 10.240.6.1 Each /23 supports 126 servers. Full building capacity: 504 dual-homed server connections.

## 10.4 Switch Configuration (PicOS)

The following template applies to each of the 4 routers. Substitute the correct VLAN, subnet, AS numbers, and VRRP priority per router.

### Router Interface and BGP (MEP-91-Core01 example)

# ─── L3 interface for server P2P links ─────────────────────────────────────

set vlans vlan-id 300 description "Server_BGP_P2P"

set vlans vlan-id 300 l3-interface "vlan-300"

set l3-interface vlan-interface vlan-300 address 10.240.0.1 prefix-length 23

# ─── Physical port to server NIC ────────────────────────────────────────────

# Note: one port per server, access mode, native VLAN 300

set interface gigabit-ethernet xe-0/1/5 family ethernet-switching port-mode "access"

set interface gigabit-ethernet xe-0/1/5 family ethernet-switching native-vlan-id 300

# ─── BGP peer-group for all servers in building ──────────────────────────────

set protocols bgp local-as 64512

set protocols bgp peer-group SERVERS remote-as 65499

set protocols bgp peer-group SERVERS allow-as-in 1

set protocols bgp peer-group SERVERS address-family ipv4-unicast enable true

# ─── Add one neighbour line per server NIC connected to this router ─────────

set protocols bgp neighbor 10.240.0.2 peer-group SERVERS description "server01-NIC1"

set protocols bgp neighbor 10.240.0.6 peer-group SERVERS description "server02-NIC1"

# ... repeat per server

# ─── VRRP — Core01 is primary (priority 120) ────────────────────────────────

set protocols vrrp interface vlan-300 vrid 300 version 3

set protocols vrrp interface vlan-300 vrid 300 ip 10.240.0.254

set protocols vrrp interface vlan-300 vrid 300 priority 120

set protocols vrrp interface vlan-300 vrid 300 load-balance disable false

### VRRP Priority Reference Across All 4 Routers

| Router / Role | VRRP Priority |
| --- | --- |
| Core01 — Primary | 120 |
| Core02 — Backup 1 | 110 |
| Res03 — Backup 2 | 100 |
| Res04 — Backup 3 | 90 |

# 11. eBGP Server Configuration

## 11.1 Overview — How the Server Connects

Each server NIC is a standalone routed interface — there is no bonding or LACP. The server runs FRR (Free Range Routing) to establish 4 independent eBGP sessions, one to each router. The loopback IP is the server’s stable identity and the only address clients ever connect to. The /30 NIC IPs are purely routing infrastructure and are never exposed in DNS.

Key Principle: The Loopback is Everything Clients connect to the server&#x2019;s loopback /32 (e.g. 10.249.0.11). DNS always resolves to the loopback — never to a NIC IP. The loopback is advertised via BGP to all 4 routers simultaneously. All 4 routers install the /32 route and ECMP load-balance inbound traffic. The server&#x2019;s NIC IPs are invisible to clients — they exist only for BGP peering.

## 11.2 Network Interface Configuration (Netplan)

Replace variables with server-specific values. The loopback /32 is the most important element — all services bind to this address.

# /etc/netplan/01-network.yaml

# Variables:

# {{ nic1_ip }} e.g. 10.240.0.2 (NIC toward Core01)

# {{ nic2_ip }} e.g. 10.240.2.2 (NIC toward Core02)

# {{ nic3_ip }} e.g. 10.240.4.2 (NIC toward Res03)

# {{ nic4_ip }} e.g. 10.240.6.2 (NIC toward Res04)

# {{ loopback }} e.g. 10.249.0.11 (server identity — unique per server)

network:

version: 2

ethernets:

eth0: # NIC1 → Core01

addresses: [{{ nic1_ip }}/23]

routes: [] # No default — BGP handles all routing

eth1: # NIC2 → Core02

addresses: [{{ nic2_ip }}/23]

routes: []

eth2: # NIC3 → Res03

addresses: [{{ nic3_ip }}/23]

routes: []

eth3: # NIC4 → Res04

addresses: [{{ nic4_ip }}/23]

routes: []

lo:

addresses:

- 127.0.0.1/8

- {{ loopback }}/32 # Service IP — clients connect here

## 11.3 FRR BGP Configuration Template

This template is identical for every server in a building. Only the router-id, NIC IPs, and loopback change. Perfect for Ansible deployment.

# /etc/frr/frr.conf

# Ansible variables:

# {{ loopback }} — unique per server e.g. 10.249.0.11

# {{ nic1_ip }} — NIC1 IP e.g. 10.240.0.2

# {{ nic2_ip }} — NIC2 IP e.g. 10.240.2.2

# {{ nic3_ip }} — NIC3 IP e.g. 10.240.4.2

# {{ nic4_ip }} — NIC4 IP e.g. 10.240.6.2

# {{ core01_ip }} — Core01 router IP e.g. 10.240.0.1

# {{ core02_ip }} — Core02 router IP e.g. 10.240.2.1

# {{ res03_ip }} — Res03 router IP e.g. 10.240.4.1

# {{ res04_ip }} — Res04 router IP e.g. 10.240.6.1

frr defaults traditional

hostname {{ inventory_hostname }}

!

router bgp 65499

bgp router-id {{ loopback }}

!

! ─── 4 router peers ────────────────────────────────────────────────────────

neighbor {{ core01_ip }} remote-as 64512

neighbor {{ core01_ip }} description Core01

neighbor {{ core02_ip }} remote-as 64513

neighbor {{ core02_ip }} description Core02

neighbor {{ res03_ip }} remote-as 64514

neighbor {{ res03_ip }} description Res03

neighbor {{ res04_ip }} remote-as 64515

neighbor {{ res04_ip }} description Res04

!

address-family ipv4 unicast

! ─── Advertise ONLY the loopback — never the /30 NIC IPs ─────────────────

network {{ loopback }}/32

!

neighbor {{ core01_ip }} activate

neighbor {{ core02_ip }} activate

neighbor {{ res03_ip }} activate

neighbor {{ res04_ip }} activate

exit-address-family

!

line vty

!

# 12. Client Connectivity

This section explains how clients reach a server that has up to 4 NICs and 4 independent BGP sessions. The design is entirely transparent to clients — from their perspective the server is a normal host with a single IP address.

## 12.1 The Core Problem and Solution

A server with 4 NICs has 4 point-to-point IPs (one per router link). Clients cannot be expected to know which IP to use, and no single NIC IP represents the server reliably — if that NIC fails, the IP becomes unreachable.

The solution is the server loopback /32. This is a virtual IP on the lo interface that is always up as long as the server is running. It is advertised via BGP to all 4 routers simultaneously. Clients connect to this single stable address regardless of which physical NICs are up or which paths are active.

How it works end-to-end 1. DNS resolves server01.immunocore.local → 10.249.0.11 (loopback) 2. Client sends traffic to 10.249.0.11 3. Campus routers have 4 equal-cost BGP paths to 10.249.0.11 (one via each router) 4. ECMP distributes inbound traffic across all 4 paths 5. Server responds from 10.249.0.11 (loopback — always up) 6. Return traffic is ECMP load-balanced across whichever NIC paths are active

## 12.2 Failure Scenarios

| Failure | Client Impact |
| --- | --- |
| 1 NIC or link fails | BGP session to that router drops. 3 paths remain. ECMP continues. Loopback IP unchanged. No client impact beyond brief BGP convergence (~1-3 sec). |
| 1 router fails | Same as above — 3 paths remain. Clients unaffected beyond BGP convergence window. |
| 2 NICs fail | 2 paths remain, ECMP continues across them. No client impact. |
| 3 NICs fail | 1 path remains. All traffic via single router. Server still reachable at same loopback IP. |
| All 4 NICs fail | Server unreachable. Only failure scenario that causes an outage. |
| Server reboots | All 4 BGP sessions drop simultaneously. Routes withdrawn from all routers. Server unreachable until BGP reconverges after boot. |

## 12.3 Asymmetric Routing

Because each NIC peers independently, inbound and outbound traffic may take different paths. A client packet may arrive via Core01 while the server response leaves via Res03. This asymmetric routing is normal and expected in this design.

Firewall Consideration If a stateful firewall sits in the path between clients and servers, it must see BOTH directions of a TCP flow to maintain state. With asymmetric routing the return path may traverse a different router and firewall instance. Mitigations: • Use stateful firewall clustering with session synchronisation across all 4 router paths • Or deploy the firewall at a point where all paths converge (above the 4 routers) • Or use connection tracking at the server level (iptables/nftables) rather than inline firewall

## 12.4 DNS Rules

| Rule | Detail |
| --- | --- |
| Always use the loopback | DNS A record must point to the server&#x2019;s /32 loopback (10.249.x.x). Never publish NIC IPs in DNS. |
| One record per server | Single A record per server. No round-robin DNS needed — ECMP at the routing layer handles load distribution. |
| Reverse DNS | PTR record for the loopback IP should resolve back to the server&#x2019;s FQDN. |
| NIC IPs | Never put in DNS. They are routing infrastructure only and will confuse clients if a NIC fails. |

## 12.5 Ansible Variable Structure

Since all 250+ servers use the same FRR template, Ansible host_vars drives all server-specific values. The group_vars hold building-level constants shared across all servers in a building.

# host_vars/server01.yml — unique per server

loopback: 10.249.0.11 # BGP router-id and client-facing IP

nic1_ip: 10.240.0.2 # NIC toward Core01

nic2_ip: 10.240.2.2 # NIC toward Core02

nic3_ip: 10.240.4.2 # NIC toward Res03

nic4_ip: 10.240.6.2 # NIC toward Res04

# group_vars/mep91_servers.yml — shared across all servers in MEP-91

bgp_server_as: 65499

core01_asn: 64512

core02_asn: 64513

res03_asn: 64514

res04_asn: 64515

core01_ip: 10.240.0.1

core02_ip: 10.240.2.1

res03_ip: 10.240.4.1

res04_ip: 10.240.6.1

# Adding a new server = create host_vars file + run playbook

# No switch config changes needed (neighbour added to peer-group)

## 12.6 Verification Commands

| Location | Command |
| --- | --- |
| Server — FRR | vtysh -c &#x27;show bgp summary&#x27; |
| Server — FRR | vtysh -c &#x27;show bgp ipv4 unicast&#x27; |
| Server — FRR | vtysh -c &#x27;show ip route&#x27; |
| Server — OS | ip route show |
| Server — OS | ip addr show lo |
| PicOS router | run show bgp neighbors |
| PicOS router | run show bgp ipv4 unicast |
| PicOS router | run show ip route bgp |
| PicOS router | run show vrrp |